ET 438a

Using Matlab to Generate Bode and Step Responses

- Start any of the computers in D106 and find the student version of Matlab under the program menu.

- Start Matlab.  After Matlab loads, click on the file menu and select New m-File.  This will start a text editor for making Matlab script files.

- Enter the following lines of code and save the file.

clear all;

close all;

n=input(‘enter the numerator coefficients: ‘);

d=input(‘enter the denominator coefficients: ‘);

sys=tf(n,d);

sys

bode(sys);

grid on;

G=1./n;

sys=sys*G

sys

figure;

step(sys);

- From the Matlab prompt, EDU>>, type the filename that you give to this script file.  This will run the script file.

- Follow the program prompts.  Enter the numerator coefficients in decreasing powers of s (e.g. s2, s1, s0).

- Enter the denominator coefficients in decreasing powers of s.

- The program will automatically produce the Bode and step responses of the defined transfer function.

- Cut and paste the graphs into Word for printing or print the results directly from Matlab.